共通 API 資訊 (WISE-4000/4200/4400/4600/2200)

- JSON Field Definitions (LogMsg Array)

| Field | Abbreviation | Data Type | Description |
| --- | --- | --- | --- |
| Periodic / Event | PE | Number | Recording mode/Event type (Refer to device event tables) |
| Timestamp | TIM | String | Storage timestamp in ISO 8601 or UTC format |
| UUID | UID | String | Universally Unique Identifier of the device |
| MAC ID | MAC | String | MAC address or LoRa EUI |
| Recording message | Record | String | Hexadecimal string containing event details |

- JSON Response Example：

{

"LogMsg": [

{

"PE": 6,

"TIM": "2014-11-11T15:48:32+08:00",

"UID": "WISE-4060_00D0C9CC0001",

"MAC": "00-D0-C9-CC-00-01",

"Record": "00000001"

},

{

"PE": 8,

"TIM": "2014-11-11T15:49:44+08:00",

"UID": "WISE-4060_00D0C9CC0001",

"MAC": "00-D0-C9-CC-00-01",

"Record": "0A090B04"

}

]

}

2. WISE-4000 & WISE-4200 Series (Wi-Fi)

Applicable Models: WISE-4000, WISE-4220, WISE-4250 11

- System Logging Event Table (PE Mapping)

| PE Value | Description | Record Parsing Logic |
| --- | --- | --- |
| 1 | Wireless Connection | Last 4 octets of the connected MAC address |
| 2 | Wireless Disconnection | Last 4 octets of the disconnected MAC address |
| 3 | Communication WDT | Byte 3: Index; Bytes 0-2: User IP (See detail table) |
| 4 | Cloud File Upload | Bytes 3-2: Index (0: I/O, 1: System); |
| 5 | Cloud Data Push | Bytes 3-2: Index (0: I/O, 1: System); |
| 6 | SNTP Status | 0: No error, 1: DNS error, 2: No socket, 3: No reply, 4: Socket fatal |
| 7 | Power Action | 1: Power on, 2: System restart, 3: Power off, 4: CoreTask WDT timeout |
| 8 | Memory Full/Overwrite | 1: IO full, 2: IO overwrite, 3: System overwrite |
| 9 | Remote Access Fail | IP address of the rejected access attempt 20 |
| 10 | Login Error | IP address associated with the failed login 21 |
| 11 | FW Upgrade | Upgraded firmware version string 22 |
| 12 | RTC Battery Low | Internal Real-Time Clock battery voltage is low 23 |
| 13 | Config Table Error | Error items indicated by bit order (0-16) (Bit Order: 0:Device Info, 1:WiFi, 2:Network, 3:Access Control, 4:IO, 5:Modbus 0X, 6:Modbus 4X, 7:User Account, 8:Internal Buffer, 9:Analog Cal, 10:IO Log, 11:Cloud, 12:File Upload, 13:Private Server, 14:System Log, 15:Internal Buffer, 16:Tag Setting.) |
| 14 | Internal Flash Error | Error accessing internal flash memory 25 |
| 15 | WiFi Event | Detailed Event ID (0x01~0x19). Includes Disconnect reasons, RSSI changes, IP acquired, Ping errors, and Socket connections |
| 16 | P2P Status | 1: Access control error, 2: Password error, 3: No QOS ACK . |
| 17 | Webserver Utility | Byte 3: Index (0x01: Delete user, 0x02: List full); Bytes 0-2: IP 26 |
| 18 | HW Error | Byte 3: 1 indicates I2C failure 27 |

- Detailed WiFi Event Table (PE 15)

When PE is 15 on a Wi-Fi device, the Record is parsed using Byte 3 as the event code 17.

| Byte 3 (Hex) | Description | Data Detail (Bytes 2, 1, 0) |
| --- | --- | --- |
| 0x01 | WLAN disconnect event | Reason code of WiFi module18. |
| 0x02 | Unexpected event | Event code of WiFi module. |
| 0x03 | Unexpected socket | Event code of WiFi module. |
| 0x04 | Tx socket failed | Byte 2: Socket ID; Byte 1-0: Reason19. |
| 0x05 | Disconnect info | First Log: Byte 2: Profile[0], Byte 1: Priority, Byte 0: Name length . Second Log (Secondary bytes): Bytes 3-0: SSID content . Byte 3: SSID[0] Byte 2: SSID[1] Byte 1: SSID[2] Byte 30: SSID[3] |
| 0x06 | RSSI level change | Byte 2: RSSI; Byte 1: Old level; Byte 0: New level21. |
| 0x07 | RSSI Histogram | First Log: Byte 2-0: RSSI_His[0] to [2]. Second Log (Secondary bytes): Bytes 2-0: RSSI_His[3] to [5]. |
| 0x08 | Unexpected WLAN policy | Byte 2: Policy; Byte 1-0: 023. |
| 0x09 | IP acquired | Byte 2-0: Acquired IP address24. |
| 0x0A | WLAN RF reset | Byte 2-0: Result code25. |
| 0x0B | Push connection fail | Error code for server connection (Push)26. |
| 0x0C | Upload connection fail | Error code for server connection (Upload). |
| 0x0D | MQTT connection fail | Error code for server connection (MQTT)27. |
| 0x0E | Device RF fatal error | Byte 2: Sender; Byte 1-0: Status28. |
| 0x0F | Device RF abort error | Byte 2: AbortType; Byte 1-0: AbortData29. |
| 0x10 | Check ping error | Byte 2: 1:None, 2:Data error; Byte 0: Gateway IP30. |
| 0x11 | Net config error | Fixed value 031. |
| 0x12 | Connection list full | Fixed value 0 (excludes webserver)32. |
| 0x13 | Reboot interval timeout | Fixed value 033. |
| 0x14 | Socket connect | Byte 2: Type (1:Modbus, 2:Cloud, 3:SNTP, 4: UDPCFG, 5:P2P, 6: WebServer ); Byte 0: Socket ID34. |
| 0x15 | Socket disconnect | Socket closure event. |
| 0x16 | RF WDT | Byte 2: WDT Mode(1: Disassociate, 2:Ping); Byte 1: Action (0:Reset, 1:Reboot, 2:Re-associate)35. |
| 0x17 | RF callback event | Byte 2: Event code36. |
| 0x18 | RF module message | Byte 2-1: Message code; Byte 0: 1:Timeout, 2:Failure37. |
| 0x19 | RF module WiFi event | Byte 2-1: Event ID; Byte 0: 038. |

3. WISE-4000/LAN Series

For WISE-4000/LAN modules, the PE field determines the category of the log entry. Below is the exhaustive list of PE codes and their parsing logic 2.

| PE | Description | Record Parsing Logic |
| --- | --- | --- |
| 1 | TCP Connection | Byte 3-2: Type (00: TCP); Byte 1: Error item; Byte 0: Content3. |
| 2 | Reserved | No data recorded 4. |
| 3 | Communication WDT | Communication Watchdog Timer event 5. |
| 4 | Cloud File Upload | 1 (Connect server error), 2 (Upload data fail) . |
| 5 | Cloud Data Push | 1 (Connect error), 2 (Push IO fail), 3 (Push System fail) |
| 6 | SNTP | 0: No error, 1: DNS error, 2: No socket, 3: No reply, 4: Socket fatal, Other: NTP time 8. |
| 7 | Power Action | 1: Power on, 2: System restart, 3: Power off 9. |
| 8 | Log Memory State | 1: IO full, 2: IO overwrite, 3: System overwrite 10. |
| 9 | Remote Access Fail | IP address of the rejected access attempt 11. |
| 10 | Login Error | IP address of the failed login attempt 12. |
| 11 | FW Upgrade | Upgraded firmware version string 13. Ex: A1.00 B01 => 0A100B01 |
| 12 | RTC Battery Low | Internal Real-Time Clock battery voltage is low. |
| 13 | Config Table Error | Error items indicated by bit order (See Section 3) 15. |
| 14 | Flash Access Error | Internal flash memory access error 16. |
| 15 | Reserved | No data recorded for LAN series 17. |
| 16 | Reserved | No data recorded for LAN series 18. |

- TLS Handshake Failure Table (PE 1)

When PE is 1 and Byte 1 is 0 (Embed TLS failure), Byte 0 indicates the specific handshake failure step24.

| Byte 0 (Hex) | Handshake Step | Byte 0 (Hex) | Handshake Step |
| --- | --- | --- | --- |
| 0 | HELLO REQUEST | 8 | CLIENT KEY EXCHANGE |
| 1 | CLIENT HELLO | 9 | CERTIFICATE VERIFY |
| 2 | SERVER HELLO | A | CLIENT CHANGE CIPHER |
| 3 | SERVER CERTIFICATE | B | CLIENT FINISHED |
| 4 | SERVER KEY EXCHANGE | C | SERVER CHANGE CIPHER |
| 5 | CERTIFICATE REQUEST | D | SERVER FINISHED |
| 6 | SERVER HELLO DONE | E | FLUSH BUFFERS |
| 7 | CLIENT CERTIFICATE | F | HANDSHAKE WRAPUP |

- Internal Configuration Error Bit Order (PE 13)

The following table provides the complete list of all 17 bits defined for the configuration error item:

| Bit | Error Item | Description |
| --- | --- | --- |
| 0 | Device Information | Errors in general device identification data. |
| 1 | Wifi Setting | Errors in wireless configuration (Shared firmware base). |
| 2 | Network Setting | Errors in IP, Subnet, or Gateway configurations. |
| 3 | Access Control Setting | Errors in IP white-list or access restrictions. |
| 4 | IO Setting | Errors in digital/analog input or output parameters. |
| 5 | Modbus 0X Address | Errors in Modbus Coil address mappings. |
| 6 | Modbus 4X Address | Errors in Modbus Holding Register address mappings. |
| 7 | User Account Setting | Errors in login credentials or permission levels. |
| 8 | Internal Buffer | Error related to the internal data buffer configuration. |
| 9 | Analog Calibration Value | Errors in the calibration offsets for analog modules. |
| 10 | IO Log Setting | Errors in the logging interval or storage conditions. |
| 11 | Cloud Setting | Errors in cloud platform connection parameters. |
| 12 | File Upload Setting | Errors in FTP/HTTP file upload configurations. |
| 13 | Private Server Setting | Errors in custom private server endpoint settings. |
| 14 | System Log Setting | Errors in the system logging configuration itself. |
| 15 | Internal Buffer | Secondary error related to the internal buffer. |
| 16 | File Upload Tag Setting | Errors in the naming or tagging of uploaded files. |

4. WISE-4671 / WISE-4471 NB-IoT Series

System Log Event Table (PE Mapping)

The PE field identifies the category of the recorded event and dictates how the Record field should be parsed 10.

| PE | Event Description | Record Parsing Logic |
| --- | --- | --- |
| 1 | Cellular Info | Records modem initialization, registration status, and RF resets (See Section 3)11. |
| 2 | Cellular CME Error | Error codes refer to specific AT Command manuals. WISE-4471: Error codes refer to SARA-R4_ATCommands_(UBX-17003787_R08) WISE-4671: Error codes refer to BG96_AT_Commands_Manual_V2.3, |
| 3 | Communication WDT | Communication Watchdog Timer events 13. |
| 4 | Cloud File Upload | Byte 3, 2: Index (0: I/O, 1: System); Byte 1, 0: Error code 14. |
| 5 | Cloud Data Push | Byte 3, 2: Index (0: I/O, 1: System); Byte 1, 0: Error code 15. |
| 6 | Cellular CMS Error | Error codes refer to specific AT Command manuals. WISE-4471: Error codes refer to SARA-R4_ATCommands_(UBX-17003787_R08), WISE-4671: Error codes refer to BG96_AT_Commands_Manual_V2.3, |
| 7 | Power Action | 1: Power on, 2: System restart, 3: Power off. |
| 8 | Memory Status | 1: IO full, 2: IO overwrite, 3: System overwrite. |
| 9 | Remote Access Success | Displays the last 6 digits of the phone number. |
| 10 | Remote Access Fail | Displays the last 6 digits of the phone number. |
| 11 | FW Upgrade | Upgraded firmware version string (e.g., A1.00 B01) . Ex: A1.00 B01 => 0A100B01 |
| 12 | Battery | Detailed battery voltage and temperature status (See Section 4) . |
| 13 | Internal Config Error | Error items defined by bit order (See Section 5) 23. |
| 14 | Internal Flash Error | Error accessing internal flash memory 24. |
| 15 | Reserved | No data recorded 25. |
| 16 | Reserved | No data recorded 26. |

- Cellular Information Details (PE 1)

When the PE value is 1, the Record string identifies the specific state of the cellular modem and network registration2.

| Code (No.) | Description / Event Status |
| --- | --- |
| 1 | Modem Initial failed: The cellular modem failed to initialize properly3. |
| 2 | Get PIN status failed: The device could not retrieve the SIM PIN status4. |
| 3 | Set packet data protocol failed: PDP context configuration failed5. |
| 4 | Set SMS service center address failed: SMSC address configuration error6. |
| 5 | Registered: The device has successfully registered to the home network7. |
| 6 | Not registered: MT is not currently searching for an operator8. |
| 7 | RF reset: A reset command was issued to the RF module9. |
| 8 | RF module reset: The physical RF hardware module has been reset10. |
| 9 | Unregistered timeout: Registration was lost due to a timeout11. |
| 10 | RF module not respond: The RF hardware is not responding to commands12. |
| 11 | Attempting Registration: Not registered, but currently searching or trying to attach13. |
| 12 | Registration denied: The network has explicitly denied the registration request14. |
| 13 | CEREG: Unknown: Registration status is unknown (Refer to 3GPP TS 27.007)15. |
| 14 | Registered, roaming: The device is registered but on a roaming network16. |
| 15 | RF module manufacturer error: An internal error reported by the RF module hardware17. |

- Battery and Charging Status (PE 12)

Note: For the WISE-4471, only event code 0 (RTC battery low) is supported28.

| No. | Error Item / Event Description |
| --- | --- |
| 0 | RTC battery low 29 |
| 1 | Battery low 30 |
| 2 | Over-Temperature detected in Charge condition 31 |
| 3 | Over-Temperature detected in Discharge condition 32 |
| 4 | Internal Short is detected 33 |
| 5 | Full-charged condition reached 34 |

- Internal Configuration Error Bit Order (PE 13)

When the PE value is 13, the Record field is a hexadecimal value that must be converted to binary. Each bit represents a specific section of the device configuration2.

| Bit | Error Item | Description |
| --- | --- | --- |
| 0 | Device information | Errors in general device identification data3. |
| 1 | Cellular setting | Errors in NB-IoT/LTE-M network parameters4. |
| 2 | User account setting | Errors in login credentials or permission levels5. |
| 3 | Access control setting | Errors in IP white-list or access restrictions6. |
| 4 | IO setting | Errors in digital/analog input or output parameters7. |
| 5 | DI counter | Errors in Digital Input counter configurations8. |
| 6 | IO log setting | Errors in the I/O data logging configuration9. |
| 7 | Cloud setting | Errors in cloud platform connection parameters10. |
| 8 | File upload setting | Errors in FTP/HTTP file upload configurations11. |
| 9 | Private server setting | Errors in custom private server endpoint settings12. |
| 10 | System log setting | Errors in the system logging configuration itself13. |
| 11 | Last address | Errors in the recorded last memory/network address14. |
| 12 | RS-485 | Errors in the RS-485 serial communication settings15. |
| 13 | Modbus RTU | Errors in the Modbus RTU protocol configuration16. |
| 14 | System mark | Errors in internal system state markers17. |
| 15 | Paas setting | Errors in Platform-as-a-Service integration settings18. |

4. WISE-4610 / WISE-2410 NB-IoT Series

System Log Event Table (PE Mapping)

The PE field identifies the category of the recorded event and dictates how the Record field should be parsed 10.

The PE field defines the category of the log. The Record field is a hexadecimal string parsed based on the PE value2.

| PE | Event Description | Record Parsing Logic |
| --- | --- | --- |
| 1 | Wireless Connection | Records LoRaWAN connection attempts and status3. |
| 7 | Power On/Off | Byte 3: Action; Bytes 0–2: Cause/Event4. |
| 8 | Memory Status | 1: IO full; 2: IO overwrite; 3: System overwrite. |
| 11 | FW Upgrade | Version string. Ex: A1.00 B01 => 0A100B01 |
| 12 | Battery Info | Byte 3: Data Index (0–4); Bytes 0–2: Value/Status7. |
| 13 | Config Error | Byte 3 (Mode): 0:Error Table (Bitmask), 1:Reset Cause . Bytes 0–2: Binary bit-order for specific configuration tables. |
| 14 | Flash Error | Byte 3 (Event): Index 1–9 defining the operation type . Bytes 0–2: Address, Length, or Sector Size (See PE 14 Detail Table). |
| 15 | RF Event | Byte 3: Protocol Index (1–7, 0x0A); Bytes 0–2: Protocol Data 10. |
| 18 | Hardware Event | Byte 3: Module Index (1–2); Bytes 0–2: Error Details11. |

- Detailed Power & Restart Logic (PE 7)

- Byte 3 (Action): 1: Power on, 2: System restart, 3: Power off, 4: CoreTask WDT timeout12.

- Bytes 0–2 (Restart Cause): Triggered when Byte 3 is 213:

| Value | Restart Cause | Value | Restart Cause |
| --- | --- | --- | --- |
| 1 | F/W image load complete 14 | 5 | Restart from LoRaWAN downlink 15 |
| 2 | RESTful Restart command 16 | 6 | Power detected during low battery 17 |
| 3 | LoRa configurations changed 18 | 7 | ASCII restart command 19 |
| 4 | RESTful Reset to Default 20 | 8 | I/O connector pin test complete 21 |

- Battery Information Table (PE 12)

The parsing of Bytes 0–2 depends on the index provided in Byte 322:

| Byte 3 | Event / Data | Bytes 0–2 Value Details |
| --- | --- | --- |
| 0 | Battery Warning | 0: Low RTC Battery, 1: Low Battery23. |
| 1 | Temperature | Current battery temperature 24. |
| 2 | Cycle count | Total battery charge cycles 25. |
| 3 | Remain capacity | Remaining battery capacity 26. |
| 4 | Battery Status | 00000001: Low Battery Event; 01XXXXXX: Voltage in mV 27. |

Ex: 00000001  Low Battery Event

Ex: 01000DB5  Battery voltage 3509 mV

- Internal Configuration Table (PE 13)

Byte 3 = 0 (Error Table)

When Byte 3 is 0, convert the record to binary. Each bit represents a configuration section:

- Bit 0: Device information.

- Bit 1: LoRa setting.

- Bit 2: IO setting.

- Bit 3: DI count.

- Bit 4: System log.

- Bit 5: RS485 setting.

- Bit 6: Modbus RTU.

- Bit 7: Position.

- Bit 8: IO log setting.

- Bit 9: Modbus RTU expansion.

Byte 3 = 1 (Reset to Default Cause)

0: Table not found; 1: Web command; 2: I/O change; 3: ASCII command.

- Internal Flash Access Error (PE 14)

When a flash access error occurs, use the value in Byte 3 of the Record field to identify the event type, then interpret Bytes 0–2 according to that event2:

| Byte 3 (Event) | Operation / Event Description | Bytes 0–2 (Content Description) |
| --- | --- | --- |
| 1 | Read | The specific Memory Address where the read operation failed3. |
| 2 | Write | The specific Memory Address where the write operation failed4. |
| 3 | Length | The Length of data that the system attempted to process5. |
| 4 | Error Length | The Error Length detected during the operation6. |
| 5 | Protect Address | The Protected Memory Address that caused an access violation7. |
| 6 | Erase | The Memory Address where the erase operation failed8. |
| 7 | Erase Size | The size of the sector being erased: 0: 32K, 1: 4K, 2: 64K9. |
| 8 | Erase Protect Address | The Protected Address triggered during an erase operation10. |
| 9 | Out of Sector | The Memory Address that was determined to be out of valid sector bounds11. |

- RF Event Primary Index (PE 15)

The following table defines the specific category of the RF event based on the value of Byte 3 in the hexadecimal Record string.

| Byte 3 (Hex) | Event Description | Data Detail (Byte 2, 1, 0) |
| --- | --- | --- |
| 01 | Join fail | Fixed value 000000222. |
| 02 | RF initial fail | Fixed value 000000333. |
| 03 | Connect fail | Byte 2-0: Status Code . |
| 04 | LoRaWAN Stack Events | Byte 2-0: Stack Event Code (Refer to LoRaWAN Stack Event Codes)5. |
| 05 | MAC Command | Header: Cmd ID + Param 1-2. Log 2+: Remaining Params. (Refer to MAC Command Reference)6. |
| 06 | Frame Counter (Fcnt) | Byte 2: 01: Uplink, 02: Downlink, 03: Difference; Byte 1-0: Value. |
| 07 | Error Event | Byte 2: Event Category; Byte 1-0: Status Code8. |
| 08 | Send Error | Same mapping logic as Byte 3 = 07. |
| 09 | Downlink Frame | Header: FC, Seq, Len. Log 2+: Raw Payload (3 bytes/log). (Refer to Downlink Frame Structure) |
| 0x0A | Data Rate Change | Byte 1: Original Data Rate; Byte 0: New Data Rate (0–6). |

- LoRaWAN Stack Event Codes (Byte 3 = 04)

| Code (Hex) | Description | Code (Hex) | Description |
| --- | --- | --- | --- |
| 000000 (0) | Connected 12 | 000007 (7) | Rx done 13 |
| 000001 (1) | Disconnected 14 | 000008 (8) | Rx timeout 15 |
| 000003 (3) | Tx timeout 16 | 000009 (9) | Rx error 17 |
| 000004 (4) | Tx error (Retries exhausted) 18 | 00000A (10) | Join failure 19 |
| 000006 (6) | Tx scheduling error 20 | 00000C (12) | Automatic uplink error 21 |

- MAC Command Detail Table (PE 15, Byte 3 = 0x05)

Below is the technical breakdown of common MAC commands and how their data is distributed across logs.

| Command ID (Byte 2) | Command Name | Total Log Records | Parameter Allocation Logic & Sequence |
| --- | --- | --- | --- |
| 0x02 | LinkCheckAns | 1 Record | Contains only the Command ID (Bytes 1–0 are typically 0/Padding). |
| 0x03 | LinkADRReq | 4 Records | Log 1: Cmd ID + DR/TXPower (Byte 1) & Redundancy (Byte 0). Logs 2–4: Sequential recording of ChMask (L/H), Resulting Settings, and ACK Status. |
| 0x04 | DutyCycleReq | 1 Record | Byte 0: MaxDutyCycle. |
| 0x05 | RXParamSetupReq | 3 Records | Log 1: Cmd ID + RX1DRoffset (Byte 1) & RX2DataRate (Byte 0). Logs 2–3: Sequential recording of Frequency (3 bytes) and ACK Status. |
| 0x06 | DevStatusReq | 1 Record | Contains only the Command ID (Bytes 1–0 are typically 0/Padding). |
| 0x07 | NewChannelReq | 3 Records | Log 1: Cmd ID + Channel Index (ChIndex) & DrRange. Logs 2–3: Sequential recording of Frequency and ACK Status. |
| 0x08 | RXTimingSetupReq | 1 Record | Byte 1: Delay setting. |
| 0x09 | TxParamSetupReq | 1 Record | Byte 1: EIRP / Dwell Time settings. |
| 0x0A | DlChannelReq | 3 Records | Log 1: Byte 2: Cmd ID + Byte 1: 00 + Byte 0: Channel Index (ChIndex). Logs 2–3: Sequential recording of Frequency-related parameters and ACK Status. |

1. LinkADRReq (Cmd ID: 0x03) — Total: 4 Logs

This command manages adaptive data rates and channel masks. It records the request parameters (Logs 1–2) and the final operational result (Logs 3–4).

| Log Order | Byte 3 | Byte 2 | Byte 1 (Content) | Byte 0 (Content) |
| --- | --- | --- | --- | --- |
| Log 1 | 0x05 | 0x03 | DataRate / TX Power | Redundancy |
| Log 2 | 0x05 | 0x03 | ChMask L | ChMask H |
| Log 3 | 0x05 | 0x03 | DataRate after setting | TxPower after setting |
| Log 4 | 0x05 | 0x03 | Status Bits (ACK) | Bytes Passed |

Log 4, Byte 1 (Status Bits) Details:

- Bits 7:3: Reserved

- Bit 2: Power ACK

- Bit 1: DataRate ACK

- Bit 0: Channel mask ACK

2. RXParamSetupReq (Cmd ID: 0x05) — Total: 3 Logs

This command configures the downlink receiving parameters for the second window.

| Log Order | Byte 3 | Byte 2 | Byte 1 (Content) | Byte 0 (Content) |
| --- | --- | --- | --- | --- |
| Log 1 | 0x05 | 0x05 | DLSettings | Status Bits (ACK) |
| Log 2 | 0x05 | Freq | Freq [15:8] | Freq [7:0] |
| Log 3 | 0x05 | 0x05 | 0 | Bytes Passed |

Log 1, Byte 0 (Status Bits) Details:

- Bit 2: RX2 DataRate ACK

- Bit 1: RX1 DR offset ACK

- Bit 0: Channel frequency ACK

3. NewChannelReq (Cmd ID: 0x07) — Total: 3 Logs

Used to add or modify a specific radio channel frequency and data rate range.

| Log Order | Byte 3 | Byte 2 | Byte 1 (Content) | Byte 0 (Content) |
| --- | --- | --- | --- | --- |
| Log 1 | 0x05 | 0x07 | ChIndex | Status Bits (ACK) |
| Log 2 | 0x05 | Freq | Freq [15:8] | Freq [7:0] |
| Log 3 | 0x05 | 0x07 | DrRange | Bytes Passed |

Log 1, Byte 0 (Status Bits) Details:

- Bit 1: DR range ACK

- Bit 0: Channel frequency ACK

4. DlChannelReq (Cmd ID: 0x0A) — Total: 3 Logs

Assigns a dedicated downlink frequency to a specific existing channel.

| Log Order | Byte 3 | Byte 2 | Byte 1 (Content) | Byte 0 (Content) |
| --- | --- | --- | --- | --- |
| Log 1 | 0x05 | 0x0A | ChIndex | Status Bits (ACK) |
| Log 2 | 0x05 | Freq | Freq [15:8] | Freq [7:0] |
| Log 3 | 0x05 | 0x0A | 0 | Bytes Passed |

Log 1, Byte 0 (Status Bits) Details:

- Bit 1: Uplink frequency ACK

- Bit 0: Channel frequency ACK

- RF Error Event Detail Table (PE 15, Byte 3 = 07 or 08)

When parsing the hexadecimal Record, if Byte 3 is 07 or 08, use this table to decode Byte 2:

| Byte 2 (Hex) | Error Category | Description of Failure |
| --- | --- | --- |
| 0x00 | Event Code | General LoRaWAN stack event error. |
| 0x01 | Initialize Error | Failure during the initial RF module setup. |
| 0x02 | Set Join Error | Failed to set parameters for the Join procedure. |
| 0x03 | Get Join Error | Failed to retrieve Join status or parameters. |
| 0x04 | Set Channel Plan Error | Error while configuring regional channel frequencies. |
| 0x05 | Get Channel Plan Error | Failed to read the current channel plan configuration. |
| 0x06 | Set Join Config Error | Error in configuring OTAA/ABP security keys. |
| 0x07 | Get Join Config Error | Failed to retrieve stored security/Join configurations. |
| 0x08 | Set Send Error | Error while preparing the transmission buffer. |
| 0x09 | Send Error | Most Common: The actual transmission failed. |
| 0x0A | Receive Error | Failure during the Rx1 or Rx2 receiving windows. |
| 0x0B | Internal Stack Error | Critical error within the LoRaWAN protocol stack. |
| 0x0C | Downlink Frame Error | Error occurred while receiving a downlink packet. |
| 0x0D | Downlink Parsed Error | Packet received but failed data parsing (See sub-codes below). |

- Downlink Frame Structure

The reconstruction follows a "Header + Data Blocks" pattern. All entries in a single downlink sequence will share the same PE (15) and the same Byte 3 (0x09).

A. The Header Entry (First Log)

This entry acts as the metadata descriptor for the incoming packet.

| Byte | Field | Description |
| --- | --- | --- |
| Byte 3 | Event Index | Fixed at 0x09. |
| Byte 2 | Frame Control (FC) | Status of the frame (e.g., Confirmed vs. Unconfirmed). |
| Byte 1 | Sequence Number (SN) | The Downlink Frame Counter from the server. |
| Byte 0 | Length (L) | Total number of bytes in the WPayload. |

B. The Data Entries (Subsequent Logs)

Every entry following the header provides exactly 3 bytes of the payload until the total Length (L) is reached.

| Byte | Field | Content |
| --- | --- | --- |
| Byte 3 | Event Index | Fixed at 0x09. |
| Byte 2 | Data Segment 1 | Payload Byte [n] |
| Byte 1 | Data Segment 2 | Payload Byte [n+1] |
| Byte 0 | Data Segment 3 | Payload Byte [n+2] |